| ===================== HOUSEKEEPING ===================== |  |  |  |
| --- | --- | --- | --- |
| erase startup-config |  |  |  |
| reload |  |  |  |
| hostname |  |  |  |
| no ip domain-lookup |  |  |  |
| ip route 0.0.0.0 0.0.0.0 g0/1 (ABQ) |  |  |  |
| line con 0 |  |  |  |
| logging synchronous |  |  |  |
| exec-timeout 0 0 |  |  |  |
| line vty 0 4 |  |  |  |
| logging synchronous |  |  |  |
| exec-timeout 0 0 |  |  |  |
| ====================== INTERFACES ====================== |  |  |  |
| int g0/0 |  |  |  |
| desc TRUNK to 3560 |  |  |  |
| no shut |  |  |  |
| int g0/0.1 |  |  |  |
| desc subif vlan 1 |  |  |  |
| encap dot1q 1 native |  |  |  |
| ip add 172.16.x.1 255.255.255.0 |  |  |  |
| no shut |  |  |  |
| int g0/0.10 |  |  |  |
| desc subif vlan 10 |  |  |  |
| encap dot1q 10 |  |  |  |
| ip add 172.16.x.1 255.255.255.0 |  |  |  |
| no shut |  |  |  |
| int g0/0.20 |  |  |  |
| desc subif vlan 20 |  |  |  |
| encap dot1q 20 |  |  |  |
| ip add 172.16.x.1 255.255.255.0 |  |  |  |
| no shut |  |  |  |
| int g0/0.30 |  |  |  |
| desc subif vlan 30 |  |  |  |
| encap dot1q 30 |  |  |  |
| ip add 172.16.x.1 255.255.255.0 |  |  |  |
| no shut |  |  |  |
| int g0/1 (ABQ) |  |  |  |
| desc WAN to Instr |  |  |  |
| ip add 172.30.20.1 255.255.255.252 |  |  |  |
| no shut |  |  |  |

| int s0/0/0 (ABQ & LA) |  |  |  |
| --- | --- | --- | --- |
| desc |  |  |  |
| ip add 172.16.255.x 255.255.255.252 |  |  |  |
| clock rate 2000000 (ABQ) |  |  |  |
| no shut |  |  |  |
| int s0/0/1 (ABQ and SEA) |  |  |  |
| desc |  |  |  |
| ip add 172.16.255.x 255.255.255.252 |  |  |  |
| clock rate 2000000 (ABQ) |  |  |  |
| no shut |  |  |  |
| ip summary |  |  |  |
| ========================= EIGRP ========================= |  |  |  |
| router eigrp 1 |  |  |  |
| no auto-summary |  |  |  |
| passive-interface g0/0 |  |  |  |
| redistribute static (ABQ) |  |  |  |
| (ABQ) |  |  |  |
| network 172.30.10.0 0.0.0.3 |  |  |  |
| network 172.16.255.0 0.0.0.3 |  |  |  |
| network 172.16.255.4 0.0.0.3 |  |  |  |
| network 172.16.255.8 0.0.0.3 |  |  |  |
| network 172.16.4.0 0.0.0.255 |  |  |  |
| network 172.16.5.0 0.0.0.255 |  |  |  |
| network 172.16.6.0 0.0.0.255 |  |  |  |
| network 172.16.7.0 0.0.0.255 |  |  |  |
| (LA) |  |  |  |
| network 172.16.255.0 0.0.0.3 |  |  |  |
| network 172.16.8.0 0.0.0.255 |  |  |  |
| network 172.16.9.0 0.0.0.255 |  |  |  |
| network 172.16.10.0 0.0.0.255 |  |  |  |
| network 172.16.11.0 0.0.0.255 |  |  |  |
| (SEA) |  |  |  |
| network 172.16.255.4 0.0.0.3 |  |  |  |
| network 172.16.0.0 0.0.0.255 |  |  |  |
| network 172.16.1.0 0.0.0.255 |  |  |  |
| network 172.16.2.0 0.0.0.255 |  |  |  |
| network 172.16.3.0 0.0.0.255 |  |  |  |
| CAUTION!! You won’t be able to ping the Instr_3560 until you set up NAT!! |  |  |  |
| But you can ping the outer interface of the ABQ router. |  |  |  |
| ========================= NAT ========================= |  |  |  |
| (ABQ) |  |  |  |
| access-list 10 permit 172.16.0.0 0.0.3.255 |  |  |  |
| access-list 10 permit 172.16.4.0 0.0.3.255 |  |  |  |
| access-list 10 permit 172.16.8.0 0.0.3.255 |  |  |  |
| ip nat inside source list 10 interface g0/1 overload |  |  |  |
| int s0/0/0 |  |  |  |
| ip nat inside |  |  |  |
| int s0/0/1 |  |  |  |
| ip nat inside |  |  |  |
| int range g0/0.1,g0/0.10,g0/0.20,g0/0.30 |  |  |  |
| ip nat inside |  |  |  |
| int g0/1 |  |  |  |
| ip nat outside |  |  |  |
| ===================== DHCP ===================== |  |  |  |
| (ABQ) |  |  |  |
| ip dhcp excluded-address 172.16.5.1 172.16.5.99 |  |  |  |
| ip dhcp pool ABQ-POOL-2 |  |  |  |
| network 172.16.5.0 255.255.255.0 |  |  |  |
| default-router 172.16.5.1 |  |  |  |
| dns-server 4.2.2.2 8.8.8.8 |  |  |  |
| exit |  |  |  |
| ip dhcp excluded-address 172.16.6.1 172.16.4.99 |  |  |  |
| ip dhcp pool ABQ-POOL-3 |  |  |  |
| network 172.16.6.1 255.255.255.0 |  |  |  |
| default-router 172.16.6.1 |  |  |  |
| dns-server 4.2.2.2 8.8.8.8 |  |  |  |
| exit |  |  |  |
| ip dhcp excluded-address 172.16.7.1 172.16.4.99 |  |  |  |
| ip dhcp pool ABQ-POOL-4 |  |  |  |
| network 172.16.7.0 255.255.255.0 |  |  |  |
| default-router 172.16.7.1 |  |  |  |
| dns-server 4.2.2.2 8.8.8.8 |  |  |  |
| exit |  |  |  |
| (LA) |  |  |  |
| ip dhcp excluded-address 172.16.9.1 172.16.9.99 |  |  |  |
| ip dhcp pool LA-POOL-2 |  |  |  |
| network 172.16.9.0 255.255.255.0 |  |  |  |
| default-router 172.16.9.1 |  |  |  |
| dns-server 4.2.2.2 8.8.8.8 |  |  |  |
| exit |  |  |  |
| ip dhcp excluded-address 172.16.10.1 172.16.10.99 |  |  |  |
| ip dhcl pool LA-POOL-3 |  |  |  |
| network 172.16.10.0 255.255.255.0 |  |  |  |
| default-router 172.16.10.1 |  |  |  |
| dns-server 4.2.2.2 8.8.8.8 |  |  |  |
| exit |  |  |  |
| ip dhcp excluded-address 172.16.11.1 172.16.11.99 |  |  |  |
| ip dhcp pool LA-POOL-4 |  |  |  |
| network 172.16.11.0 255.255.255.0 |  |  |  |
| default-router 172.16.11.1 |  |  |  |
| dns-server 4.2.2.2 8.8.8.8 |  |  |  |
| exit |  |  |  |
| (SEA) |  |  |  |
| ip dhcp excluded-address 172.16.1.1 172.16.1.99 |  |  |  |
| ip dhcp pool SEA-POOL-2 |  |  |  |
| network 172.16.1.0 255.255.255.0 |  |  |  |
| default-router 172.16.1.1 |  |  |  |
| dns-server 4.2.2.2 8.8.8.8 |  |  |  |
| exit |  |  |  |
| ip dhcp excluded-address 172.16.2.1 172.16.2.99 |  |  |  |
| ip dhcp pool SEA-POOL-3 |  |  |  |
| network 172.16.2.0 255.255.255.0 |  |  |  |
| default-router 172.16.2.1 |  |  |  |
| dns-server 4.2.2.2 8.8.8.8 |  |  |  |
| exit |  |  |  |
| ip dhcp excluded-address 172.16.3.1 172.16.3.99 |  |  |  |
| ip dhcp pool SEA-POOL-4 |  |  |  |
| network 172.16.3.0 255.255.255.0 |  |  |  |
| default-router 172.16.3.1 |  |  |  |
| dns-server 4.2.2.2 8.8.8.8 |  |  |  |
| exit |  |  |  |
| ========== Set up the switches now! ========== |  |  |  |
| ===Test with Ping while watching nat translations on ABQ. |  |  |  |
| ================= EIGRP Authentication ================== |  |  |  |
| Note: key chain not supported in Packet Tracer! |  |  |  |
| key chain EIGRP (EIGRP is the name of the key chain) |  |  |  |
| key 1 |  |  |  |
| key-string cisco |  |  |  |
| int s0/0/0 |  |  |  |
| ip authentication mode eigrp 1 md5 |  |  |  |
| ip authentication key-chain eigrp 1 EIGRP |  |  |  |
| int s0/0/1 |  |  |  |
| ip authentication mode eigrp 1 md5 |  |  |  |
| ip authentication key-chain eigrp 1 EIGRP |  |  |  |
| ================== Route Summarization ================== |  |  |  |
| int s0/0/0 |  |  |  |
| ip summary-address eigrp 1 172.16.8.0 255.255.252.0 (LA) |  |  |  |
| ip summary-address eigrp 1 172.16.4.0 255.255.252.0 (ABQ) |  |  |  |
| int s0/0/1 |  |  |  |
| ip summary-address eigrp 1 172.16.8.0 255.255.252.0 (SEA) |  |  |  |
| ip summary-address eigrp 1 172.16.4.0 255.255.252.0 (ABQ) |  |  |  |
| ===================== Frame Relay ======================= |  |  |  |
| int s0/0/1 |  |  |  |
| encap frame-relay |  |  |  |
| frame-relay interface-dlci 101 |  |  |  |
| no keepalive |  |  |  |
| exit |  |  |  |
| ========================= SSH ========================= |  |  |  |
| enable secret cisco |  |  |  |
| ip domain-name cisco.com |  |  |  |
| crypto key generate rsa (512) |  |  |  |
| username cisco secret cisco |  |  |  |
| line vty 0 4 |  |  |  |
| no transport input all |  |  |  |
| transport input ssh |  |  |  |
| login local |  |  |  |
| ========================= ACLs ========================= |  |  |  |
| (ABQ) |  |  |  |
| access-list 101 permit tcp 172.16.4.0 0.0.3.255 host 192.168.115.9 eq 80 |  |  |  |
| access-list 101 permit tcp 172.16.4.0 0.0.3.255 host 192.168.115.211 eq 80 |  |  |  |
| access-list 101 permit tcp 172.16.4.0 0.0.3.255 host 192.168.112.23 eq 80 |  |  |  |
| access-list 101 deny tcp 172.16.4.0 0.0.3.255 any eq 80 |  |  |  |
| access-list 101 permit ip any any |  |  |  |
| access-list 20 permit host 172.16.7.100 |  |  |  |
| int range g0/0.1,g0/0.10,g0/0.20,g0/0.30 |  |  |  |
| ip access-group 101 in |  |  |  |
| line vty 0 4 |  |  |  |
| access-class 20 in |  |  |  |
| (LA) |  |  |  |
| access-list 101 permit tcp 172.16.8.0 0.0.3.255 host 192.168.115.9 eq 80 |  |  |  |
| access-list 101 permit tcp 172.16.8.0 0.0.3.255 host 192.168.115.211 eq 80 |  |  |  |
| access-list 101 permit tcp 172.16.8.0 0.0.3.255 host 192.168.112.23 eq 80 |  |  |  |
| access-list 101 deny tcp 172.16.8.0 0.0.3.255 host any eq 80 |  |  |  |
| access-list 101 permit ip any any |  |  |  |
| access-list 20 permit host 172.16.7.100 |  |  |  |
| int range g0/0.1,g0/0.10,g0/0.20,g0/0.30 |  |  |  |
| ip access-group 101 in |  |  |  |
| line vty 0 4 |  |  |  |
| access-class 20 in |  |  |  |
| (SEA) |  |  |  |
| access-list 101 permit tcp 172.16.0.0 0.0.3.255 host 192.168.115.9 eq 80 |  |  |  |
| access-list 101 permit tcp 172.16.0.0 0.0.3.255 host 192.168.115.211 eq 80 |  |  |  |
| access-list 101 permit tcp 172.16.0.0 0.0.3.255 host 192.168.112.23 eq 80 |  |  |  |
| access-list 101 deny tcp 172.16.0.0 0.0.3.255 any eq 80 |  |  |  |
| access-list 101 permit ip any any |  |  |  |
| access-list 20 permit host 172.16.7.100 |  |  |  |
| int range g0/0.1,g0/0.10,g0/0.20,g0/0.30 |  |  |  |
| ip access-group 101 in |  |  |  |
| line vty 0 4 |  |  |  |
| access-class 20 in |  |  |  |